# Standard safe operating practices for cable skidder operators

- Personal protective equipment

Hardhat and high-visibility clothing to be worn when not inside the cab.

Safety boots in good condition, properly laced, must be worn at all times. Worn-out soles and heels could lead to slips and falls. Chainsaw operators must also have cut-resistant boots that meet CSA standards.

Eye protection will be worn where there is danger of falling or flying debris from equipment or loads, especially in windy conditions. Chainsaw operators must also use a face screen.

Hand protection will be worn when handling cable or any other material where there is danger of cuts or puncture injury.

Hearing protection will be worn when exposed to noise levels exceeding 85dBA.

Leg protection is required by all chainsaw operators. All protection should be rated to a minimum of 3600 rpm chain speed. It must also provide protection for the back of the legs. (BNQ standard CAN/BNQ 1923-450-M91 Leg Protective Devices for Chain Saw Users)

- Mounting and dismounting – three-point contact will be used to mount and dismount equipment.

- Inspection and repairs – skidders will be inspected prior to use to ensure good mechanical condition.

When working under or around skidders, for inspections or repairs, the skidder must be locked and tagged out, and immobilized and secured against inadvertent movement.

- Housekeeping – cabs, steps, windshields, windows and mirrors must be kept clean at all times. All debris should be removed.

- Parking – the skidder must be parked on level ground, clear of hazards, to allow ease of access. The blade must be lowered to the ground or fully supported.

- Travelling – proper gear selection must be used to maintain control. Drive according to terrain conditions. Side-hill travel is to be avoided whenever possible and straight-line winching is to be used as much as possible.

- Danger zone – danger zone is defined as the area around operating machines or working personnel in which there is potential for being struck by moving equipment or objects. The danger zone may vary according to the machine or work being performed. Operators must make sure that all persons, vehicles and equipment are clear of the danger zone before the vehicle or its components are moved.

- Lockout – lockout procedures must be followed during mechanical service, repairs or inspection for the protection of employees and equipment.

Refer to company and manufacturer’s procedures on lockout.

Communications while assisting in mechanical repairs – When operators are assisting mechanics to repair machines, clear communications must be established prior to starting the tasks. The operator and the mechanic must each know who will be responsible for:

a) Starting or moving a machine

b) Ensuring that anyone involved is in a clear and safe position

c) Directing the movement of the machine

d) Ensuring that it is safe to resume working and that all guards are in place.

The operator must have a clear understanding of what is to be done and follow the specific lockout procedure and instructions given by the mechanic responsible for performing the job.

- Skidding – wood must be piled as per company requirements.

- Fuelling – shut off the engine while fuelling. No smoking. Be aware of slip and trip hazards.

Beware of spills and splash-back. Return hose to its proper storage position when fuelling is completed.

- Hazardous materials – read WHMIS label. If there is no label, contact the supervisor. Refer to MSDS if further information is needed.

Use protective equipment and follow safe handling instructions as outlined on WHMIS label.

If an incident occurs, follow first aid instructions.

Use proper storage procedures.

- Chainsaw requirements – must be equipped with: operating chain brake, safety throttle, chain catcher, hand guard, spark arrestor and low kickback chain.

Must be filed according to the manufacturer’s specifications and maintained in a safe operating condition.

Must be started on the ground, on a stump or with the leg-lock method.

Must be held firmly with both hands while cutting and have the chain stopped when finger is off the throttle.

When carried with one hand, bar must be backwards and brake engaged (or engine turned off).

- Felling – before felling the desired tree, all spring poles and chicots in the felling area are to be carefully lowered to the ground.

Lower chicots with the skidder fairlead whenever possible. Or carefully fell with a chainsaw in its natural fall direction.

No trees are to be felled unless everyone is clear of the danger zone.

The base of the tree and at least one escape route (back and away at 45 degrees) are to be cleared of obstacles before felling.

Natural fall direction must first be established (lean, wind, heavy branches, ice or snow loading), then desired direction of fall based on skidding pattern, canopy openings, terrain, etc. Then cut the notch in the desired direction of fall.

Conventional notches are acceptable on most trees. Humboldt notches are required on trees with veneer potential.

Wedges are to be carried at all times and used when required to assist in directional felling.

Hang-ups and free-standing trees are to be lowered to the ground immediately and before cutting another tree.

The escape route must be used as the tree falls. Do not turn your back on the falling tree.

- Limbing, butting, topping and bucking – before commencing limbing operations, stay away from a fallen tree until all hazards have been assessed.

Limbing and topping is to be done only after all hazards have been addressed.

Limb, top and buck trees with feet on a solid surface, in a balanced stance, and on the uphill side of the tree.

Limb, top and buck trees after determining compression/tension characteristics.

Limbs are to be removed flush to the stem of the tree.

Butt ends must be reasonably flush with hinge snipe removed.

- Skidding – wood must be piled as per company requirements.

Skidding is to be done at a proper speed and with minimal damage to standing trees.

Operator must establish location of cutter before entering felling area and identify problem trees that need to be removed first.

Before leaving the cutting area, the skidder operator is to get an all-clear signal from the cutter.

- Safeguards – manufacturer’s safeguards (warning labels, window screens, brakes, handholds, steps, engine covers, etc.) are to be maintained. If safeguards are defective, report to supervisor immediately.

By signing below, both parties agree that all of the above items have been discussed and understood

Employee: ______________________ Supervisor: __________ Date: _____________