# Standard safe operating practices for 
feller buncher operators

- Personal protective equipment

Hardhat and high-visibility clothing to be worn when not inside the cab.

Safety boots in good condition, properly laced, must be worn at all times. Worn-out soles and heels could lead to slips and falls.

Eye protection will be worn where there is danger of falling or flying debris from equipment or loads, especially in windy conditions.

Hand protection will be worn when handling cable or any other material where there is danger of cuts or puncture injury.

Hearing protection will be worn when exposed to noise levels exceeding 85dBA.

- Mounting and dismounting – three-point contact will be used to mount and dismount equipment.

Before leaving the seat, the operator must ensure that the boom is on the ground and the brake is engaged.

While walking on the feller buncher, especially in areas without handrails, or on the ground around the feller buncher, workers must take extra caution and identify slip and trip hazards.

- Inspection, maintenance or repairs – daily inspections of equipment should be carried out at the beginning and at the end of shifts. Defects must be reported promptly for repair and recorded on the condition report.

Daily maintenance must be carried out, including greasing and cleaning. Cabs must be swept out daily and decks cleaned at the end of every shift.

When working under or around the feller buncher, for inspections or repairs, the feller buncher must be locked and tagged out, and immobilized and secured against inadvertent movement.

- Housekeeping – cabs, steps, windshields, windows and rear-view mirrors must be kept clean at all times. All debris should be removed.

- Parking the feller buncher – the feller buncher must be parked on level ground, clear of hazards, to allow ease of access. The boom should be positioned so the door can be used as designed before the machine is shut off.

- Travelling – proper gear selection must be used to maintain control up and down hills. If the feller buncher is being pulled, both machines must be kept in the same gears.

- Danger zone – danger zone is defined as the area around operating machines or working personnel, in which there is potential for being struck by moving equipment or objects. The danger zone may vary according to the machine or work being performed. Operators must make sure that all persons, vehicles and equipment are clear of the danger zone before the vehicle is moved.

Any person approaching a machine must remain clear of the danger zone until the attention of the operator is obtained through some method of communication (for example, radio contact or visual contact).

- Lockout – lockout procedures must be followed during mechanical service, repairs or inspection for the protection of employees and equipment.

Refer to company and manufacturer’s procedures on lockout.

Communications while assisting in mechanical repairs – When operators are assisting mechanics to repair machines, clear communications must be established prior to starting the tasks. The operator and the mechanic must each know who will be responsible for:

a) Starting or moving a machine

b) Ensuring that anyone involved is in a clear and safe position

c) Directing the movement of the machine

d) Ensuring that it is safe to resume working and that all guards are in place.

The operator must have a clear understanding of what is to be done and follow the specific lockout instructions and instructions given by the mechanic responsible for performing the job.

- Wood placement for skidding – wood bundles must be placed in orderly fashion with bridging, if possible, and in suitable sizes to accommodate the terrain to ensure ease of skidding and in some cases, limbing and toping.

- Hazard trees – chicots and dislodged trees must be lowered when encountered. Stump heights must be left as low as the terrain will allow.

- Fuelling – shut off the engine while fuelling. No smoking. Be aware of slip and trip hazards.

Beware of spills and splash-back. Return hose to its proper storage position when fuelling is completed.

Before entering the danger zone for the purpose of fuelling a machine, ensure that communication (radio/eye contact) with the operator has been made. The machine should be shut off before the fuelling vehicle approaches. The fuelling vehicle must be out of the danger zone before the machine is started.

When a fuel truck is used for refuelling, have the driver pass the hose up or, if alone, put the hose up onto catwalk and get up on machine by steps or tracks.

- Hazardous materials – read WHMIS label. If there is no label, contact the supervisor. Refer to MSDS if further information is needed.

Use protective equipment and follow safe handling instructions as outlined on WHMIS label.

If an incident occurs, follow first aid instructions.

Use proper storage procedures.

- Blowdown – harvesting – Take the top tree first, if possible. If not, pull a tree free rather than lifting against obstacles. Cut all trees as close to the root system as possible. If a long stump is left after the first cut, re-cut it closer to the ground.

Keep the work area as free of large debris as possible. Swing uprooted trees to the side before cutting the stump off, where possible, to allow easy access to bundles. Cut all species as you come to them. Pay special attention to springpole and spear hazards. Inspect the cutting head, boom and swing assembly twice per shift. Accumulate trees according to boom capacity. Avoid allowing butts to rub excessively on saw blade due to high fire risk.

By signing below, both parties agree that all of the above items have been discussed and understood

Employee: ______________________ Supervisor: ___________ Date:_____________