# Standard safe operating practices for fuel truck drivers

- Personal protective equipment

- Hardhat and high-visibility clothing to be worn when not inside the cab.

Safety boots in good condition, properly laced, must be worn at all times. Worn-out soles and heels could lead to slips and falls.

Eye protection will be worn where dispensing fuel and there is danger of falling or flying debris from equipment or loads, especially in windy conditions.

Hand protection will be worn to protect against splashing or when handling cable or any other material where there is danger of cuts or puncture injury.

Hearing protection will be worn when exposed to noise levels exceeding 85dBA.

- Mounting and dismounting – three-point contact will be used to mount and dismount equipment.

- Inspection and repairs – fuel trucks will be inspected prior to use to ensure good mechanical condition. Emergency supplies including a spill kit, first aid kit, eye wash, and fire extinguisher must be available with the truck, and checked regularly.

When working under or around fuel trucks, for inspections or repairs, the fuel truck must be locked and tagged out, and immobilized and secured against inadvertent movement.

- Housekeeping – cabs, steps, windows, windshields, and mirrors must be kept clean at all times. All debris should be removed.

- Parking – the fuel truck must be parked on level ground, clear of hazards, to allow ease of access, at least 300 feet from any water.

- Travelling – proper gear selection must be used to maintain control. Drive according to road conditions.

Ensure all hoses, valves, and latches are properly stowed and secured.

Drivers must ensure the TDG requirements are met, including placards and shipping documents. Drivers must also have proof of TDG training.

- Danger zone – danger zone is defined as the area around operating machines or working personnel, in which there is potential for being struck by moving equipment or objects. The danger zone may vary according to the machine or work being performed. Operators must make sure that all persons, vehicles and equipment are clear of the danger zone before the vehicle is moved.

- Lockout – lockout procedures must be followed during mechanical service, repairs or inspection for the protection of employees and equipment.

Refer to company and manufacturer’s procedures on lockout.

Communications while assisting in mechanical repairs – When operators are assisting mechanics to repair machines, clear communications must be established prior to starting the tasks. The operator and the mechanic must each know who will be responsible for:

a) Starting or moving a machine

b) Ensuring that anyone involved is in a clear and safe position

c) Directing the movement of the machine

d) Ensuring that it is safe to resume working and that all guards are in place.

The operator must have a clear understanding of what is to be done and follow the specific lockout instructions and instructions given by the mechanic responsible for performing the job.

- Fuelling – shut off the engine while fuelling. No smoking. Be aware of slip and trip hazards.

Beware of spills and splash-back. Return hose to its proper storage position when fuelling is completed.

- Hazardous materials – read WHMIS label. If there is no label, contact the supervisor. Refer to MSDS if further information is needed.

Use protective equipment and follow safe handling instructions as outlined on WHMIS label.

If an incident occurs, follow first aid instructions.

Use proper storage procedures.

By signing below, both parties agree that all of the above items have been discussed and clearly understood.

Employee: _________________ Supervisor: __________________ Date: __________