# Standard safe operating practices for chipper operators

- Personal protective equipment

Hardhat and high-visibility clothing to be worn when not inside the cab.

Safety boots in good condition, properly laced, must be worn at all times. Worn-out soles and heels could lead to slips and falls.

Eye protection will be worn where there is danger of falling or flying debris from equipment or loads, especially in windy conditions.

Hand protection will be worn when handling cable or any other material where there is danger of cuts or puncture injury.

Hearing protection will be worn when exposed to noise levels exceeding 85dBA.

Respiratory protection should be worn when cleaning the machine or when dust levels are high.

- Mounting and dismounting – three-point contact will be used to mount and dismount equipment.

- Inspection and repairs – chippers will be inspected prior to use to ensure good mechanical condition.

When working under or around chippers, for inspections or repairs, the chipper must be locked and tagged out, and immobilized and secured against inadvertent movement.

- Housekeeping – cabs, steps, windows, windshields, and mirrors must be kept clean at all times. All debris should be removed.

- Parking – the chipper must be parked on level ground, clear of hazards, to allow ease of access.

- Travelling – proper gear selection must be used to maintain control. Drive according to terrain conditions

- Danger zone – danger zone is defined as the area around operating machines or working personnel, in which there is potential for being struck by moving equipment or objects. The danger zone may vary according to the machine or work being performed. Operators must make sure that all persons, vehicles and equipment are clear of the danger zone before the chipper is moved.

Chippers also have a danger zone around the flail chain housing. The safe procedure, according to the manufacturer, must be followed to ensure the safety of the worker working on the flail chains. This includes access to the flail chains through the infeed roller.

- Lockout – lockout procedures must be followed during mechanical service, repairs or inspection for the protection of employees and equipment.

Refer to company and manufacturer’s procedures on lockout.

Communications while assisting in mechanical repairs – When operators are assisting mechanics to repair machines, clear communications must be established prior to starting the tasks. The operator and the mechanic must each know who will be responsible for:

a) Starting or moving a machine

b) Ensuring that anyone involved is in a clear and safe position

c) Directing the movement of the machine

d) Ensuring that it is safe to resume working and that all guards are in place.

The operator must have a clear understanding of what is to be done and follow the specific lockout instructions and instructions given by the mechanic responsible for performing the job.

- Guarding – the operator needs to be aware of the pinch points in and around the chipper. All pinch points must be guarded.

- Fuelling – shut off the engine while fuelling. No smoking. Be aware of slip and trip hazards.

Beware of spills and splash-back. Return hose to its proper storage position when fuelling is completed.

- Hazardous materials – read WHMIS label. If there is no label, contact the supervisor. Refer to MSDS if further information is needed.

Use protective equipment and follow safe handling instructions as outlined on WHMIS label.

If an incident occurs, follow first aid instructions.

Use proper storage procedures.

- Moving the chipper – follow company policies regarding moving the chipper. Ensure everyone is out of the danger zone before moving it. Watch for all hazards, including overhead hazards.

- Log or chipper and sliver plug-ups – company and manufacturer safe procedures must be followed when clearing plug-ups and jam-ups.

- Fall protection must be worn at or above the height of 3 metres, including when on top of the chipper.

By signing below, both parties agree that all of the above items have been discussed and understood.

Employee: __________________ Supervisor: __________________ Date: _________